House Lumber Takeoff

Framing material checklist, takeoff methods, and formulas

A build-order reference for taking off the lumber and framing materials in a house. Each system lists the items to quantify, the material options to price, and the takeoff method or formula that turns a measurement into an order quantity. The output is a supplier-ready material list, so every count rounds up to whole pieces.

## Contents

# How to use this document

- Work top to bottom. Sections are in build order, from foundation to roof to exterior.

- Repeat the Floor System and Wall System templates once per level. A three-story house runs the floor template for the first, second, and third floors, and the wall template for each of those levels plus the basement.

- For each item, take the measurement the formula asks for (a length, an area, a count), apply the formula, then round up. Quantities are always whole pieces, sheets, rolls, or boxes.

- Assign a material to every item you are pricing. Where several options are listed, pick the one on the plans.

- Items marked Plies have no spacing formula. You size the member by span and multiply by the number of plies (x2, x3, x4) shown on the drawings.

# Conventions

| Convention | Rule |
| --- | --- |
| Round up | Every quantity uses CEILING to a whole unit. You cannot order 12.3 studs, so it becomes 13. |
| Waste factor | Applied after the raw count, then rounded up again: pieces = CEILING(raw x (1 + waste)). Typical: studs 10 percent, plates and joists 5 percent, sheathing 10 percent, roofing 10 to 15 percent. |
| Board feet | BF = (nominal thickness in x nominal width in x length ft) / 12. Used for pricing by board foot and for the by-weight nail method below. |
| Stock length rounding | A single-piece member (stud, joist, rafter, header) is cut from the smallest stock length that covers it. Joists and rafters round up to the nearest 2 ft. |
| On center (OC) | Spacing between member centerlines. 16 in OC and 24 in OC are the common cases. |
| Plies | Built-up members (beams, girders, headers, columns) are counted as N plies: a triple 2x10 beam is 3 plies. No spacing formula applies; size by span, then multiply by plies. |

# Takeoff methods

Every item in this document uses one of these six methods. The system tables name the method and give the specific formula.

| Method | Use it for | Formula |
| --- | --- | --- |
| Spacing count | Members spaced along a run: studs, joists, rafters, ceiling joists, cripples, strapping. | Qty = CEILING(Run_ft x 12 / Spacing_in) + 1, then waste. Piece length = span to next stock length. |
| Run length | Continuous runs cut from stock: plates, rim, sill, ridge and hip boards, fascia, ledger, blocking. | Qty = CEILING(Total_LF / Stock_len_ft) x plies, then waste. |
| Plies (no formula) | Built-up members: beams, girders, headers, columns. | Qty = plies (x2, x3, x4). Piece length = span (+ bearing for headers) to next stock length. |
| Coverage area | Sheet and rolled goods: sheathing, subfloor, decking, house wrap, felt, siding, shingles. | Qty = CEILING(Area_sqft / Coverage_per_unit), then waste. |
| Count | One per location: columns, posts, hangers, sonotubes, vents, pocket-door kits, hardware. | Qty = Count x multiplier. |
| Fasteners | Nails, screws, adhesive. | See Nails, fasteners, and adhesive below. |

# Nails, fasteners, and adhesive

Fasteners are estimated one of two ways, then converted to the unit they are sold in.

## Method A, sheet goods (by nail count)

Sheathing, subfloor, and decking are nailed on a set pattern, so estimate by the sheet: Nails = Sheets x Nails_per_sheet. A 6 in edge / 12 in field pattern on a 4x8 sheet is about 60 nails. Convert to purchase units with Boxes = CEILING(Total_nails / Nails_per_box).

## Method B, framing (by weight)

Plates, studs, joists, rafters, and blocking are estimated by weight from the framing board feet: Pounds = CEILING(Framing_BF / 100 x 1.25), roughly 1.25 lb of framing nails per 100 board feet. Convert with Boxes = CEILING(Total_lb / Lb_per_box).

## Adhesive

Subfloor and decking adhesive: Tubes = CEILING(Sheets / 2.5), about one 28 oz tube per 2 to 3 sheets at a 3/8 in bead.

## Fastener type guide

| Fastener | Used on | Sold as |
| --- | --- | --- |
| 3-1/4 in (16d) framing | Studs, plates, beams, blocking | Box by count or by weight (lb) |
| 2-3/8 in (8d) sheathing | Wall and roof sheathing, subfloor | Box by count |
| Joist hanger / structural | Hangers, hurricane ties, straps | Box; match to the connector |
| Ring-shank 2 in | Subfloor with adhesive, decking | Box by count |
| Roofing 1-1/4 in | Underlayment, starter, shingles | Box by count |

# Foundation and sill

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Sill seal / termite shield | Foam sill seal 3-1/2 in, 5-1/2 in; copper or aluminum termite shield | Run length CEILING(Perimeter_ft / Roll_len_ft) | One continuous run around the perimeter. |
| Sill plates (PT) | 2x4, 2x6, 2x8 SP PT | Run length CEILING(Perimeter_ft / Stock_len_ft) | PT for concrete contact. Match width to the wall. |
| Sill plates (interior) | 2x4, 2x6, 2x8 SPF #2 | Run length CEILING(Total_LF / Stock_len_ft) | Non-PT where not on concrete. |
| Anchor bolts / hardware | Anchor bolts, washers, straps | Count CEILING(Perimeter_ft / Bolt_spacing_ft) + corners | Add one within 12 in of each corner and plate end. |

# Floor system (repeat per level)

Run this template for each floor: first, second, third, and any deck-style floor. Joist span is the clear distance the joists cross; joist length rounds up to the nearest 2 ft.

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Rim / band joist | 2x8, 2x10, 2x12 SPF #2 | Run length CEILING(Perimeter_ft / Stock_len_ft) | Same depth as the joists. |
| Floor joists | 2x8, 2x10, 2x12 SPF #2; I-joists | Spacing count CEILING(Span_ft x 12 / OC) + 1 | Length to nearest 2 ft. Double under parallel partitions (x2). |
| Floor beams / girders | Built-up 2x SPF #2; LVL 1-3/4 in | Plies x2, x3, x4 | Size by span. No spacing formula. |
| Floor blocking | 2x SPF #2 (joist depth) | Run length CEILING(Rows x Building_width_ft / Stock_len_ft) | One row per code / mid-span. |
| Floor bridging | Metal or wood cross bridging; solid blocking | Count Sets = Joists per row x Rows | One row each 8 ft of span, typical. |
| Floor hardware | Joist hangers, connectors | Count One per joist / beam connection | Match to joist size. |
| Floor sheathing (subfloor) | 3/4 in T&G plywood or OSB 4x8 | Coverage area CEILING(Area_sqft / 32) | 32 sqft per sheet. Add 10 percent waste. |
| Floor adhesive | Subfloor adhesive 28 oz | Fasteners CEILING(Sheets / 2.5) | One tube per 2 to 3 sheets. |
| Subfloor nails | Ring-shank 2 in | Fasteners Sheets x 60, then boxes | With adhesive. |

# Wall system (repeat per level and type)

Run this template for each wall type on each level: exterior, interior bearing, and interior non-bearing. Wall LF is the total length of that wall type; stud length is the wall height rounded to the next stock length.

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Studs | 2x4, 2x6 SPF #2 (exterior often 2x6) | Spacing count CEILING(Wall_LF x 12 / OC) + 1 | Add studs for each corner, partition, and opening. Length = wall height to next stock. |
| Plates | 2x4, 2x6 SPF #2; PT bottom on slab | Run length CEILING(Wall_LF / Stock_len_ft) x 3 | Double top + single bottom = 3 plate runs. PT bottom on concrete. |
| Window and door headers | 2x10, 2x12 SPF #2; LVL | Plies x2, x3 + jacks | Length = opening + 0.25 ft bearing to next stock. See Openings note. |
| Dropped beams / girders | Built-up 2x; LVL | Plies x2, x3, x4 | Size by span. |
| Columns / posts | 4x4, 6x6 PT; built-up 2x; steel | Count One per location | Under beam bearing points. |
| Wall sheathing | OSB 7/16 in, plywood 4x8 | Coverage area CEILING(Wall_area_sqft / 32) | 32 sqft per sheet. |
| Wall blocking | 2x SPF #2 | Run length CEILING(Backing_LF / Stock_len_ft) | Fire blocking, backing, nailers. |
| Wall bracing | Let-in metal strap or structural panel | Count Per braced-wall line | Per plan / code. |
| Pocket doors | Pocket-door frame kit | Count One kit per opening | Kit sized to the door. |
| Wall hardware | Hold-downs, straps, connectors | Count Per location | Per plan. |

## Openings (header, king, trimmer, cripple)

| Piece | Method | Formula |
| --- | --- | --- |
| Header | Plies | Qty = plies (x2 typical). Length = Opening_ft + 0.25 ft bearing, to next stock length. |
| King studs | Count | Qty = 2 per opening (full height). |
| Trimmer / jack studs | Count | Qty = 2 per opening; add plies for wide spans. |
| Cripple studs | Spacing count | Qty = CEILING(Opening_ft x 12 / OC). Above header and below a window sill. |

# Stairs

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Stringers | 2x12 SPF #2; LVL | Count Typically 3 to 4 per flight | One per 16 in of stair width, plus one. |
| Treads | Tread stock; 5/4 or 2x | Count Risers - 1 | Nosed stock as specified. |
| Landing framing | Same as floor system | Spacing count CEILING(Span_ft x 12 / OC) + 1 | A small floor. |
| Landing sheathing | 3/4 in T&G 4x8 | Coverage area CEILING(Area_sqft / 32) | 32 sqft per sheet. |
| Stair hardware | Stringer hangers, angle brackets | Count Per connection | Match to stringer. |

# Ceiling

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Ceiling joists | 2x6, 2x8, 2x10 SPF #2 | Spacing count CEILING(Span_ft x 12 / OC) + 1 | Length to nearest 2 ft. Double under loads (x2). |
| Ceiling beams | Built-up 2x; LVL | Plies x2, x3, x4 | Size by span. |
| Ceiling rim | 2x SPF #2 (joist depth) | Run length CEILING(Perimeter_ft / Stock_len_ft) | Around the ceiling. |
| Strongbacks | 2x4 + 2x on edge | Run length CEILING(Run_LF / Stock_len_ft) | Stiffens long joist spans. |
| Strapping | 1x3 or 1x4 | Spacing count CEILING(Room_len_ft x 12 / OC) + 1 | Crosses the joists, 16 in OC. |
| Ceiling blocking | 2x SPF #2 | Run length CEILING(Backing_LF / Stock_len_ft) | Backing and fixtures. |
| Ceiling hardware | Hangers, connectors | Count Per connection | Per plan. |

# Roof

Rafter and truss counts spread across the building length; each plane is counted separately, so multiply by the number of planes. Roof area is the actual sloped area of all planes.

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Roof rafters | 2x8, 2x10, 2x12 SPF #2 | Spacing count (CEILING(Run_ft x 12 / OC) + 1) x Planes | Length by run and pitch, to next stock. |
| Trusses (alternative) | Engineered trusses (custom) | Spacing count CEILING(Length_ft x 12 / OC) + 1 | Ordered as custom units, usually 24 in OC. |
| Ridge boards / beams | 2x, LVL, or built-up | Run length / Plies CEILING(Ridge_LF / Stock) or plies | Board for rafters, beam if load-bearing. |
| Hip and valley boards | 2x, LVL | Run length / Plies CEILING(Hip_LF / Stock) | By sloped length. |
| Valley cleats | 2x SPF #2 | Run length CEILING(Valley_LF / Stock_len_ft) | Nailer for valley sheathing. |
| Collar ties | 2x SPF #2 | Count CEILING(Rafter_pairs / 2) | Every other pair, typical. |
| Roof blocking / bracing | 2x SPF #2 | Run length CEILING(Run_LF / Stock_len_ft) | Per plan / code. |
| Roof sheathing | CDX 1/2 in, OSB 4x8 | Coverage area CEILING(Roof_area_sqft / 32) | Add 10 to 15 percent for hips and valleys. |
| Sub-fascia | 2x SPF #2 | Run length CEILING(Eave_LF / Stock_len_ft) | At the eave. |
| Soffit / rake framing | 2x SPF #2 | Run length CEILING(Run_LF / Stock_len_ft) | Lookouts and outriggers. |
| Underlayment / felt | #30 felt; synthetic | Coverage area CEILING(Roof_area_sqft / Coverage_per_roll) | Felt about 200 sqft per roll after overlap. |
| Ice and water shield | Self-adhered membrane | Coverage area CEILING((Eave_LF + Valley_LF) x Width_ft / Roll_sqft) | Eaves and valleys. |
| Drip edge | Metal drip edge 10 ft | Run length CEILING((Eave_LF + Rake_LF) / 10) | 10 ft pieces. |
| Shingles | Architectural shingles | Coverage area CEILING(Roof_area_sqft / 100) squares, + waste | 1 square = 100 sqft = 3 bundles. |
| Starter / hip and ridge | Starter strip; hip and ridge caps | Run length CEILING((Eave + Rake + Hip + Ridge_LF) / Bundle_LF) | By linear foot of edge. |
| Ridge vent | Ridge vent 4 ft sections | Run length CEILING(Ridge_LF / Section_len_ft) | Along the ridge. |
| Roof flashing | Step, wall, pipe flashing | Count Per penetration / wall | Per plan. |
| Roof hardware | Hurricane ties, connectors | Count One per rafter / truss | Per plan / code. |

# Siding and exterior trim

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| House wrap | House wrap 9 ft x 100 ft | Coverage area CEILING(Wall_area_sqft / 900) | About 900 sqft per roll. |
| Siding | Vinyl, clapboard, shingle, vertical | Coverage area CEILING(Wall_area_sqft / Coverage_per_unit), + waste | By square or by piece. 10 percent waste typical. |
| Corner trim | Inside and outside corner posts | Run length CEILING(Corners x Wall_height_ft / Stock_len_ft) | Count corners x height. |
| Watertable / band trim | 1x or composite | Run length CEILING(Run_LF / Stock_len_ft) | Horizontal bands. |
| Frieze / fascia trim | 1x, 5/4, composite | Run length CEILING((Eave + Gable_LF) / Stock_len_ft) | Eave and gable. |
| Soffit trim | Vented and solid soffit | Coverage area CEILING(Soffit_area_sqft / Coverage_per_unit) | By area or LF. |
| Window and door flashing | Flashing tape, head flashing | Count Per opening | One set per opening. |
| Window and door trim | Casing / brickmould | Run length CEILING(Opening_perimeter_LF / Stock_len_ft) | Per opening perimeter. |
| Vents | Gable and soffit vents | Count Per location | Per plan. |
| Column / beam enclosure | Wrap stock, PVC | Run length CEILING(Enclosure_LF / Stock_len_ft) | Wraps posts and beams. |

# Exterior deck

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Deck joists | 2x8, 2x10 PT | Spacing count CEILING(Span_ft x 12 / OC) + 1 | Length to nearest 2 ft. |
| Deck beams | Built-up 2x PT | Plies x2, x3 | Size by span. |
| Deck ledger | 2x PT (joist depth) | Run length CEILING(Ledger_LF / Stock_len_ft) | Flashed to the house. |
| Deck rim | 2x PT (joist depth) | Run length CEILING(Perimeter_ft / Stock_len_ft) | Around the frame. |
| Deck blocking | 2x PT | Run length CEILING(Rows x Width_ft / Stock_len_ft) | Mid-span. |
| Decking | PT, cedar, composite | Coverage area CEILING(Deck_area_sqft / Board_coverage), + 10 percent | Coverage = board face width. Add waste. |
| Railing system | Post, rail, baluster kits | Run length CEILING(Rail_LF / Section_len_ft) | Plus posts by count. |
| Deck stair stringers | 2x12 PT | Count 3 to 4 per flight | By stair width. |
| Deck columns / posts | 6x6, 4x4 PT | Count One per footing | Under beam bearing. |
| Sonotubes | Concrete form tubes | Count One per footing | Per plan. |
| Deck hardware / flashing | Hangers, post bases, ledger flashing | Count Per connection | Match to member. |
| Deck trim / lattice | Skirt trim, lattice panels | Coverage area CEILING(Skirt_area_sqft / Panel_sqft) | Skirting. |

# Miscellaneous

| Item | Material options | Takeoff method and formula | Plies / notes |
| --- | --- | --- | --- |
| Bulkhead | Framing lumber + sheathing | Count Per location | Frame and sheathe as a small assembly. |
| Access / attic hatch | Framing + panel | Count Per location | Frame the rough opening. |

Standard rules of thumb are a starting point, not an engineered design. Verify spans, header and beam sizes, ply counts, and connections against local code and the project structural drawings before ordering.